W9EHQ  SECRET SQUIRREL BAOFENG

PROGRAMMING SEQUENCE

-----------------------------------------------------

The example here will be to program the DARC 2mtr rptr into memory chan. 50.

Buttons pressed will be in RED.   I am sure there more ways to skin a cat, but this works for me.

Turn on radio

With  VFO/MR  select “freq. mode”

With  A/B, move cursor to upper display

With  BAND  select VHF or UHF (in this case…VHF)

With  NUMERIC KEYPAD  enter desired RECEIVE freq. (in this case  145.430 )

Press  MENU  + 2  +  MENU

With  UP/DOWN ARROWS  select HIGH or LOW POWER

Press  MENU  +  EXIT  (radio voice confirms selection)

Press  MENU  +  5  +  MENU

With  UP/DOWN ARROWS  select WIDE

Press  MENU  +  EXIT  (radio voice confirms selection)

Press  MENU  +  1  +  3  +  MENU

With  UP/DOWN ARROWS  select PL FREQ.  (in this case, 107.2)

Press  MENU  +  EXIT  (radio voice confirms selection)

Press  MENU  +  2  +  5  +  MENU

With  UP/DOWN ARROWS  select OFSET DIRECTION  (in this case  - )

Press  MENU  +  EXIT  (radio voice confirms selection)

Press  MENU  + 2  +  6  +  MENU

With  NUMERIC KEYPAD  enter OFSET FREQ  (in this case  00.600)

Press  MENU  +  EXIT  (radio voice confirms selection)

Press  MENU  +  2  +  7  +   MENU

With  NUMERIC KEYPAD  select memory channel location  (in this case  50 )

Press  MENU  +  EXIT   (radio voice confirms RECEIVING memory channel)

With  NUMERIC KEYPAD  enter desired TRANSMIT freq.  (in this case  144.830)

Press  MENU + 2 + 7 + MENU + MENU + EXIT  (radio voice confirms XMIT freq)

Programming complete for this channel. Now to display what you have done…..

With  VFO/MR  select Channel mode in upper display

(1) Press  A/B  to move cursor to upper display

Press  UP/DOWN  ARROWS to select Channel # desired

(2) Press  A/B  to move cursor to lower display

Press  UP/DOWN ARROWS to select same channel # as displayed in upper display

(this Chan. # will be the small # to the right of the Freq. display)

So when the Channel # (top display) and the channel # next to freq. (bottom display)

agree, you now know what freq is in that particular memory channel.

Note: items 1 & 2 above can be reversed if you forget what is in each memory channel but know the freq. you want.

Also note: Assuming your channel# (upper display) and corresponding freq. for that channel (lower display) match, with the cursor is next to the channel #, the freq. displayed on the bottom display will be the RECEIVE freq. ,even when you transmit.  However, if the cursor is next to the bottom freq. display, and an offset freq. has been programmed, when you transmit, the TRANSMIT freq. will be displayed.

Good Luck  de  W9EHQ